## BY ORDER OF THE COMMANDER 36TH WING (PACAF)

36 WING INSTRUCTION 91-202

XX MAY 2017

Safety ANDERSEN AFB SAFETY PROGRAM

COMPLIANCE WITH THIS PUBLICATION IS MANADATORY

| ACCESSIBILITY: | Publications and forms are available on the e-Publishing website at www.e-Publishing.af.mil for downloading or ordering |
| --- | --- |
| RELEASABILITY: | There are no releasability restrictions on this publication |

OPR:   36 WG/SEG	Certified by: 36 WG/SE

(Lt Col Russell D. Gohn)

Pages: 52

This instruction establishes the Andersen AFB (AAFB) Safety Program, provides the commander’s policy on mishap prevention, the functional manager’s and squadron commander’s responsibilities for mishap prevention and the safety staff’s role in mishap prevention.  It implements DODI 6055.07, Mishap Notification, Investigation, Reporting, and Record Keeping, Air Force Policy Directive 91-2, Safety Programs, and Air Force Instructions 91-202, The US Air Force Mishap Prevention Program, 91-203, Air Force Consolidated Occupational Safety Instruction, 91-204, Safety Investigations and Reports, 91-207, The US Air Force Traffic Safety Program, and 90-801, Environment, Safety, and Occupational Health Council (ESOHC), and 90-802, Risk Management, as supplemented.  It establishes mishap prevention program requirements, assigns responsibilities for program elements, and contains program management information.  This instruction is applicable to all activities, organizations, associate units, tenant units, deployed units, contractors, military and DoD civilian personnel and their dependents on AAFB and areas under jurisdiction of the Commander, 36th Wing.

Ensure that all records created as a result of processes prescribed in this publication are maintained in accordance with AFMAN 33-363, Management of Records, and disposed of in accordance with the Air Force Records Disposition Schedule (RDS) located at  https://www.my.af.mil/afrims/afrims/afrims/rims.cfm/.  Refer recommended changes and questions about this publication to the Office of Primary Responsibility (OPR) using the AF IMT 847, Recommendation for Change of Publication; route AF IMT 847s from the field through the appropriate functional’s chain of command.

## SUMMARY OF CHANGES

This document is substantially revised and must be completely reviewed.

|  | 1. | Purpose. .................................................................................................................. | 4 |
| --- | --- | --- | --- |
|  | 2. | Unit Safety Representative (USR) and Additional Duty Weapon Safety Representative (ADWSR) Responsibilities. .......................................................... | 4 |
|  | 3. | Unit Safety Program Assessments and Inspections. .............................................. | 5 |
|  | 4. | Unit Motorcycle Safety Representative (MSR) Responsibilities. .......................... | 6 |
|  | 5. | Safety Meetings. .................................................................................................... | 6 |
|  | 6. | Mishap Reporting Procedures. ............................................................................... | 6 |
|  | 7. | Unit Safety and Risk Management (RM). ............................................................. | 8 |
|  | 8. | Reflective Gear Wear. ............................................................................................ | 9 |
|  | 9. | Walking, Jogging, and Running Safety. ................................................................ | 9 |
|  | 10. | Military Formations. .............................................................................................. | 9 |
|  | 11. | Bicycle, Skateboard, Roller-Skate, and Roller-Blade Safety. ................................ | 10 |
|  | 12. | Water Safety. ......................................................................................................... | 10 |
|  | 13. | Snorkeling. ............................................................................................................. | 12 |
|  | 14. | SCUBA Diving. ..................................................................................................... | 12 |
|  | 15. | Boating Safety. ...................................................................................................... | 12 |
|  | 16. | Jungle Safety. ………............................................................................................. | 13 |
|  | 17. 18. | Motorized Vehicles……………………………………………….......................... Motorcycle, moped, Low Speed Vehicle (LSV), and All-Terrain Vehicle (ATV) Operation | 13 13 |
|  | 19. | Government Vehicle Other (GVO). .……………………………………………. ........................................................................................ | 13 |
|  | 20. | Safety Awards Program. ..……………………………………………………..… | 15 |

Attachment 1—GLOSSARY OF REFERENCES AND SUPPORTING INFORMATION	17

Attachment 2—OCCUPATIONAL SAFETY MANAGEMENT BOOK	18

Attachment 3—UNIT WEAPONS SAFETY MANAGEMENT BOOK	21

Attachment 4—USR PROGRAM ASSESSMENT CHECKLIST	22

Attachment 5—ADWSR PROGRAM CHECKLIST	39

Attachment 6—CIVIL ENGINEER WORK ORDER REQUEST PROCEDURES	45

Attachment 7—OCCUPATIONAL SAFETY AND HEALTH ADMINISTRATION

(OSHA) RECEPTION PROCEDURES	48

Attachment 8—GUAM WATER SAFETY POLICY MEMORANDUMS	49

- Purpose.

  - The purpose of the 36th Wing Safety Program is to protect AAFB populous from death, injuries or illnesses by managing risks both on- and off-duty and minimize loss of Air Force resources.  This program applies to all operations except where otherwise prescribed.  The purpose of this instruction is to provide AAFB-specific guidance in order to maintain the 36th Wing’s mishap prevention program IAW Air Force Policy Directive 91-2, Safety Programs, and Air Force Instructions 91-202, The US Air Force Mishap Prevention Program, 91-203, Air Force Consolidated Occupational Safety Instruction, 91-204, Safety Investigations and Reports, 91-207, The US Air Force Traffic Safety Program, and 90-801, Environment, Safety, and Occupational Health Council.

  - Airmen are AAFB’s number one resource.  Consequently, safety is vital to ensuring mission accomplishment.  Commanders and supervisors at all levels must understand the importance of safety and how it directly relates to mission success.  Commanders must establish proactive mishap prevention programs both on- and off-duty.  Supervisors must facilitate safety education and training to ensure a safe work environment while encouraging people to actively identify and correct existing hazards.  While the ultimate safety responsibility belongs to the commander, safety is the shared responsibility of every person within the unit.  Safety is as much a personal choice as it is professional.

# Unit Safety Representative (USR) and Additional Duty Weapon Safety Representative (ADWSR) Responsibilities.

  - USRs serve as the unit’s primary point of contact for matters pertaining to safety, fire prevention, and occupational health.  USRs should be knowledgeable in the mission and activities of the unit and have a minimum of one year retainability at the time of selection.

  - ADWSRs serve as the unit’s primary point of contact for managing, facilitating, and training all unit personnel in the unit’s weapons safety program when the unit has a mission requirement to store, handle, or transport explosives.  ADWSRs should have a minimum of 1 year retainability at the time of selection.

  - Each unit shall have a primary and alternate USR and ADWSR, if applicable. USR and ADWSRs shall receive training from the 36th Wing Safety office or through a course approved by the PACAF Safety office.  Primary and Alternate USRs and ADWSRs for active duty units shall complete initial training within 30 calendar days of appointment.

    - Air Reserve Component and National Guard Primary and Alternate USRs and ADWSRs shall complete initial training within two unit training assemblies of appointment.  The unit is responsible for maintaining USR/ADWSR training records.  USRs and ADWSRs are responsible for managing the squadron commander’s occupational and weapons safety program respectively and accomplishing the following duties:

    - USRs/ADWSRs shall develop a process to ensure safety and RM topics are discussed and documented throughout the organization at least monthly and maintain for one year.

    - USRs and ADWSRs shall maintain an Occupational Safety Management Book (OSMB) IAW Attachment 2 and a Unit Weapons Safety Management Book (UWSMB) IAW Attachment 3, respectively.

    - USRs/ADWSRs shall maintain safety bulletin boards.  USRs/ADWSRs shall post a safety bulletin board in a conspicuous location within each unit facility or major work center.  If space limitations do not permit the use of a bulletin board dedicated only to safety information, post materials in segregated sections of other unit bulletin boards. As a minimum, the USRs/ADWSRs shall post the following items:

      - AFVA 91-209, Occupational Safety and Health Program.

      - CA 10 (What a Federal Employee Should Do If Injured at Work), if work center has civil service personnel.

      - Unit mishap notification procedures.

      - Instructions for completing AF Form 457 and blank forms.

      - 36th Wing Commander Safety Policy.

      - Current mishap prevention information (posters, flyers, safety grams, etc).

      - 36th Wing Safety office Point of Contact information.

      - Bioenvironmental Engineering (BE) Industrial Hygiene surveys shall be posted for a minimum of 10 days.

      - OSHA Form 300A (1 February – 30 April annually).

  - Primary, alternate USRs/ADWSRs, or a designated representative shall attend USR/ADWSR safety meetings as required.  This meeting is the primary forum for providing USRs/ADWSRs information on changes in safety policies and standards, discussing mishap prevention processes, and providing additional training.

  - USRs shall meet quarterly with the unit commander to review program requirements including the following topics:  reportable mishaps and timely reporting of those mishaps, open inspections items, open hazard reports, RM integration, and current status of the high risk activities (HRA) program.  This review must be documented to reflect all areas covered.  Utilization of the Unit Safety Representative Quarterly Report to the Unit Commander (Figure A2.1.) is highly encouraged.

# Unit Safety Program Assessments and Inspections.

  - In accordance with AFI 91-202, AFMAN 91-201, AFI 91-203, and Title 29 Code of Federal Regulations 1960.26, the 36th Wing Safety office is required to perform annual occupational and weapons safety program assessments and facility inspections of all assigned and tenant units.

  - The 36th Wing Safety office shall publish a yearly inspection schedule and provide it to USRs, ADWSRs, and commanders prior to the new fiscal or calendar year.

  - USRs and ADWSRs shall track corrective actions on all open hazards and program deficiencies identified during 36th Wing Safety office occupational and weapons safety program assessments and facility inspections.

  - USRs and ADWSRs shall submit follow-ups to the 36th Wing Safety office at 36WG.SEG2@us.af.mil within 30 calendar days of receipt of the inspection report(s).  The corrective actions shall detail the initial corrective plan of action for all reported discrepancies and shall be signed by the unit commander or designated representative.  After the initial follow-up, all other follow-ups thereafter are due every 30 days until all corrective actions are completed.  Unit commander signature is not required after completion of initial follow-up; however, USR and ADWSRs shall inform the commander on all submitted corrective actions until closure.

  - Should an inspected unit’s initial corrective plan of action require 36th Civil Engineering Squadron (CES) support, the aforementioned follow-up actions shall include resulting CE Work Order Tracking Number(s) and estimated/desired completion date(s).  Step by step directions on how to obtain CE Work Orders for follow-ups are provided in Attachment 8.

# Unit Motorcycle Safety Representative (MSR) Responsibilities.

  - Ensure all riders within the unit are identified upon arrival and information is updated in the Motorcycle Unit Safety Tracking Tool (MUSTT) database.  Verify licensing and training of the member.  If members require training, visit the 36th Wing Occupational Safety SharePoint or contact the 36th Wing Occupational Safety office at  36WG.SEG2@us.af.mil.

  - Ensure riders receive the Commander’s Initial Motorcycle briefing.

  - Maintain MUSTT database ensuring each active duty rider is accounted and updated.

  - Ensure all riders receive an annual pre-season motorcycle riders’ safety briefing.

# Safety Meetings.

  - The Environmental, Safety, and Occupational Health Council (ESOHC) serves as a forum to discuss occupational safety and health issues, advises the installation commander on occupational safety and health related matters, and makes recommendations to the commander on resolution of occupational safety and health problems.  The council is conducted semiannually and chaired by the 36th Wing Commander or Vice Wing Commander.

  - AAFB ESOHC members include 36th Wing group commanders, wing staff representatives from each two-letter office, 36th Civil Engineering Fire Emergency Services, Environmental, Public Health, Bioenvironmental Engineering, and all tenant organizations.  Joint Region Marianas Safety office representatives are extended courtesy invitations to attend AAFB ESOHC meetings, but are not AAFB ESOHC members.

  - IAW AFI 91-202, paragraph 8.3.12, written procedures to that define how to handle OSHA representative(s) during official installation visits or inquiries are found in Attachment 7.

# Mishap Reporting Procedures.

  - Unit commanders, USRs, and ADWSRs are responsible for developing and publicizing mishap notification procedures within their unit.  As a minimum requirement, an AF Form 978 will be completed by the work center supervisor or injured personnel's supervisor and returned to the safety office within five (5) workdays following the mishap or notification of the mishap, whichever is earlier.

  - Per AFI 91-202, a mishap is an “unplanned occurrence, or series of occurrences, that results in damage or injury and meets Class A, B, C, D and E mishap reporting criteria IAW AFI 91-204.  Damage or injury includes: damage to DoD property or equipment; environmental damage; occupational illness to DoD military or civilian personnel; injury to DoD military personnel on- or off-duty; injury to on-duty DoD civilian personnel; damage to public or private property, or injury or illness to non-DoD personnel caused by Air Force operations.”

  - DODI 6055.07, Mishap Notification, Investigation, Reporting, and Record Keeping, governs mishap class criteria for all the DoD service components to include guidance provided in AFI 91-204.

  - For all on-/off-duty military and on-duty DoD civilian personnel mishaps which occur during normal duty hours, USRs/ADWSRs shall report the mishap to the 36th Wing Safety Office at DSN 366-SAFE (7233) immediately following mishap notification to the unit’s commander and the 36th Wing Command Post.

  - For all on-/off-duty military and on-duty DoD civilian personnel mishaps which occur after normal duty hours and do not result in death, hospitalization, or government property damage estimated to be greater than $50,000, USRs shall report the mishap to the 36th Wing Safety Office at 366-SAFE (7233) by the close of the first duty day following the mishap.

  - For all on-/off-duty military and on-duty DoD civilian mishaps which occur after normal duty hours and result in death, hospitalization, or government property damage estimated to be greater than $50,000, USRs shall contact the 36th Wing Safety office immediately following notification to the unit’s commander and the 36th Wing Command Post.  After normal duty hours, the 36th Wing Safety office on-call person can be reached through the 36th Wing Command Post at DSN 366-2981.  Of note, all Class A and B mishaps are OPREP-3 reportable (i.e. once the 36th Wing Command Post is notified of a Class A or B mishap, the 36th Wing Command Post is required to submit a 36th Wing Commander approved OPREP-3 message within one hour).

  - The 36th Medical Group shall provide patient injury and treatment logs to the 36th Wing Safety office weekly for military and DoD civilian personnel.

  - The 36th Wing Staff Judge Advocate shall notify the 36th Wing Safety office of any claims involving injury to persons or damage to private property as a result of an Air Force operation.

  - The 36th Security Forces Squadron shall provide copies of AF Form 1315, Accident Report, when requested, and make the daily blotter available for review by the 36th Wing Safety Office staff.

  - The 36th Civil Engineer Squadron shall provide cost assessments of damage to government property as a result of natural phenomena to the 36th Wing Safety office.

  - The AAFB Fire Department shall provide copies of fire incident reports to the 36th Wing Safety Office, when requested.

  - The 36th Wing Command Post shall notify the 36th Wing Safety office of the following:

    - Activation of the Crisis Action Team (CAT) or Emergency Operations Center (EOC).

    - On-duty accidents/mishaps involving injury to USAF personnel (military or DoD civilian employees).

    - Off-duty accidents/mishaps involving injury to US Armed Forces personnel.

    - Property damage to facilities, aircraft, vehicles or equipment.

    - Accidents/mishaps involving explosives handling or storage.

  - The 36th Logistics Readiness Squadron (LRS) shall provide, on a monthly basis, cost estimates on all reported damage to Air Force motor vehicles.

# Unit Safety and Risk Management (RM).

  - Commanders or supervisors shall ensure all personnel are briefed on the PACAF HRA program regardless of their participation in high-risk activities.

    - Each individual should be surveyed, and if it is determined they are actively engaged or may engage in an HRA they should meet one on one with their commander or supervisor.  AF Form 4391, High-Risk Activities Worksheet, will be used to document the discussion and intended HRAs.  Ideally implemented, an HRA interview is designed to determine the mental and physical readiness and risk assessment of participants before an HRA occurs.

    - Further information about the Air Force HRA program may be found in AFI 91-202.  The PACAF HRA Program can be accessed via the PACAF/SE SharePoint	website	located	at:  https://hickam.eis.pacaf.af.mil/HQ%20PACAF/Staff/SE/Occupational_Safety/SitePages/Home.aspx?RootFolder=%2FHQ%20PACAF%2FStaff%2FSE%2FOccupational%5FSafety%2FShared%20Documents%2FHigh%2DRisk%20Activities&FolderCTID=0x012000E11A3416EF7D7F4FA0AF5EEA2183F6B7&View=%7BCAB7EAA2%2D3644%2D4653%2D8709%2DEFD35A23C352%7D.

7.2.  Commanders or supervisors shall provide a pre-departure safety discussion or briefing to all active duty members prior to departing work for a weekend, pass, leave, or TDY.   For all active duty members below the age of 26, it is recommended this pre-departure safety discussion or briefing be documented on AF Form 4392, Pre-Departure Safety Briefing Form if the member is leaving the local area on leave or TDY.

  - Commanders and supervisors shall apply RM while performing unit events such as challenging physical fitness sessions.

  - Supervisors shall ensure all assigned personnel are aware of local Guam hazards on- and off-duty.  Supervisors are responsible for ensuring their Airmen read and acknowledge the 36th Wing Guam Water Activity Restrictions by signing the memorandum (Attachment 8).

  - Personnel shall practice sound RM at all times.  The Air Force RM program has proven successful, and not only protects you, but also enables the 36th Wing to maintain its focus on mission readiness.  Furthermore, Air Force RM education, information, and training is available through the Air Force Safety Center Portal via the AF Portal.

    - All Air Force personnel will be trained in Air Force Risk Management Fundamentals via an option outlined in AFI 90-802, 11.3.

    - Another great resource available to all is “Check 3, GPS,” www.check3gps.com.  Check 3 GPS is a simple RM tool that can be employed in all on- and off-duty activities.  This concept helps ensure you have the right “G – Gear,” “P – Plan,” and “S – Skill” for the activity.  By ensuring these three simple steps are carefully thought through members can eliminate any unnecessary risks.

# Reflective Gear Wear.

  - Reflective gear, visible from the front and back, shall be worn while performing cardiovascular exercise (e.g. walking, jogging, running, bicycle riding, etc.) on the installation during hours of darkness, reduced visibility, or inclement weather (e.g. fog, heavy rain, etc.).  The Air Force official physical training (PT) uniform is considered reflective and does not requires additional reflective gear.

  - AAFB military, DoD civilian employee, or contractor personnel conducting activities in a traffic environment, working on or off the flightline, during hours of darkness or reduced visibility, or working as part of a construction activity shall wear reflective gear.

  - During real-world force protection conditions Charlie and Delta the wear of reflective gear shall be at the unit commander’s discretion.

# Walking, Jogging, and Running Safety.

  - Anyone performing cardiovascular exercise (e.g. walking, jogging, running, etc.) on roadways shall travel in single file against the flow of traffic (i.e. on the left side of the road).  It is highly recommended that walkers, joggers, and runners remain on the sidewalks as much as possible.  If walking, jogging or running on the road and no traffic is present, walkers, joggers, and runners may run two abreast.  At the first sign of approaching traffic, walkers, joggers, and runners shall return to single file and clear the roadway as much as possible.

  - Jogging/Running is not authorized in any areas adjacent to or along road construction areas.

  - Wearing headphones while performing cardiovascular exercise (e.g. walking, jogging, running, etc.) on roadways is not permitted.

Wearing earbuds/headphones (both ears) on base sidewalks, the Arc Light path, the base track, the rubberized/concrete sidewalk path adjacent to the Palm Tree Golf Course, and jogging pathways in the grass/dirt areas along roadways is permitted.  Both earbuds/headphones shall be removed when crossing a roadway intersection or traversing through parking lots.

  - 36th Wing military personnel who jog off the installation during hours of darkness, reduced visibility or inclement weather are highly encouraged to wear reflective gear visible from the front and back.

# Military Formations.

  - Military formations are defined as an organized body of personnel numbering at least ten maintaining formation integrity.  These formations shall have one leader and traffic guards positioned at the front and rear of the formation.

  - It is highly encouraged the leader and traffic guards shall wear (green/orange) reflective vests to be immediately recognizable.

  - If performing PT during hours of darkness, it is highly encouraged that the leader and each road guard carry a coned traffic wand flashlight.

  - Vehicles shall not enter an intersection until a formation has completely passed through and the road guards have recovered with the formation.

  - Unless a road is closed to traffic, formations shall be limited to allow for safe vehicle passing.  This restriction is not applicable to wing runs.

  - Formations shall not run on Carolinas Ave, Santa Rosa Blvd, Arc Light Blvd or Plumeria Blvd from Andersen Middle School to the AAFB back gate.  This restriction is not applicable to wing runs.

# Bicycle, Skateboard, Roller-Skate, and Roller-Blade Safety.

  - All personnel riding bicycles, coaster, roller skates, in-line skates, skateboards, and other similar devices on AAFB will wear an approved helmet.  Helmets must meet the American National Standards Institute (ANSI) or Snell Memorial Foundation requirements.

  - Play wagons, carts, small two-wheel bicycles, tricycles, roller-blades, roller-skates or skateboards may be used in parking lots and other areas during non-peak traffic hours.  Skaters must skate with the flow of traffic on roadways.  Roller-blading, roller-skating and/or skateboarding is prohibited in parking lots or roadways during hours of darkness or limited visibility.

  - Please refer to 36 WGI 31-218, Andersen AFB Motor Vehicle Traffic Supervision, for additional guidance on bicycle, skateboard, roller-skate, and roller-blade safety.

# Water Safety.

  - Guam’s waters offer an excellent opportunity for recreational enjoyment, but they also require understanding of local hazards to maintain personnel safety.  For swimming and snorkeling, the safest areas are those beaches protected from the ocean by a barrier reef.  Inside this reef, the water may be calmer, shallower and have a lighter current.   However, this calm water can be deceitful as it may lure people to venture out near or onto the reef into deeper water in the pursuit of fish or sight-seeing.  This can be extremely dangerous because of two primary dangers in swimming too close or onto the reef.

    - The first danger is caused by waves breaking on the reef.  Even in the calmest of conditions, it is difficult and dangerous to be near the reef.  The coral is extremely sharp and can cause deep and painful cuts.  There are also large cracks and holes in the coral, which can catch a foot or hand and trap a person.  Naturally, larger waves increase the danger, and in the past have proven fatal to swimmers.

    - The second danger is the current caused by waves.  In almost all cases where the waves are larger than one foot, a strong current runs along the reef.  This current is created when water forced inside the reef by the waves is higher than the sea level outside the reef.  The water will flow along the reef or shoreline until it finds a place to flow back to the sea.  This current can be very strong and cause a rip current.  A person swimming too close to the reef's edge can become caught in the current and be swept onto or over the reef.  Water activities are permitted on AAFB in designated areas only.

    - To mitigate these hazards Joint Region Marianas and the 36th Wing have developed water safety restrictions to ensure that risk is managed appropriately.  The memorandums can be found in Attachment 8 and further defined below.

  - Water restrictions on Andersen AFB are as follows:

    - Wingman concept (minimum 2 swimmers) is required for all water-related activities.

    - Reef walking is prohibited.

    - Swimming is prohibited under “red flag” conditions.

    - Swimming and water activities at Tarague Beach are permitted ONLY in the buoyed area from 0630 to 1830 hours.

    - SCUBA diving is prohibited on Andersen AFB.  This excludes commercial, diver certified off-shore boat diving.

    - Water entry to knee deep while fishing is only allowed from the Eastern (Tarague entrance) point of Tarague to the Western point (Castro/ Pangelinan Land).  Patron must have rod and reel in hand while in the water.  Water entry in all other areas including within the Tarague buoy area is not permitted while fishing.  Contact 36 FSS Outdoor Recreation for further information.  Complete fishing instructions are found in 36 WGI 34-102, Andersen Air Force Base Beach Recreation Areas and 36 WGI 32-7003, Conservation and Management of Natural Resources.

  - The following restrictions apply on Guam (off AAFB) for all Air Force personnel assigned to AAFB.   All military personnel on Guam must comply with Joint Region Marianas Notice 1620 (Attachment 8).  Individuals not under 36th Wing control will be referred to their chain of command for appropriate administrative action:

    - Cliff diving along the coastline is prohibited (reference PACAF HRA guide).

    - Water entry is prohibited at Shark’s Hole in Shark’s Cove, north of Tanguisson Beach.

    - Reef Walking is prohibited.

    - Swimming under “red flag” conditions is prohibited.

    - Swimming or fishing in the vicinity of the reef/breakwater is prohibited.  SCUBA Diving through this area is allowed if certified or in training for certification.

    - Cave diving/Underwater exploration is prohibited.  SCUBA Diving is allowed in these areas if properly certified.  Seek local dive instructor guidance before diving caves, caverns, reefs, wreckage etc.

      - Underwater exploration applies to swimming with or without SCUBA gear in underwater environments where there is a possibility of entrapment or restricted egress, meaning the swimmer/diver cannot swim vertically to the surface or exit the underwater environment (e.g., caves, caverns, reefs, wreckage, etc.).  This can also be referred to as underwater spelunking.

  - Do not engage in recreational water activity during Typhoon Conditions of Readiness (TCOR) I, II, and III, or in any area where a hazardous surf warning is in effect.

  - Maintain a constant, direct line of sight with your Wingman, fellow swimmers, and family while in the water.

  - It is highly recommended for all to wear protective footwear in the water and around the beach.

  - For further Guam water safety tips, please visit the U.S Coast Guard Sector Guam Water Safety website at http://www.uscg.mil/d14/sectguam/safety.asp.

13.  Snorkeling.

# 13.1. Snorkeling is a very popular water activity off the coast of Guam.  There are numerous locations throughout the island to enter the water, but not all are suitable for recreational snorkeling.  Never snorkel alone and always let someone know where you plan to snorkel and when you plan to return.

# 13.2. Look for signage to identify recreational snorkeling areas or beaches that are staffed with a lifeguard.  These areas are typically much safer and provide a much more pleasant experience.  Stay away from the coastal reef and beware of dangerous wildlife.

# 13.3. Proper gear is essential.  The following gear is recommended when snorkeling:  mask, snorkel, fins/booties, rash guard, gloves and a personal floatation device (PFD).  Ensure all gear is serviceable and fits properly before entering the water.

# SCUBA Diving.

  - SCUBA Divers shall obtain valid training and certification prior to engaging in SCUBA diving on Guam.  Supervisors shall track this requirement by utilizing the PACAF HRA program.

  - SCUBA Divers shall always have a dive plan and dive with a designated buddy.  Dive plans shall be communicated with a responsible person ashore or with the boat captain for boat dives.

  - SCUBA Divers shall conduct all SCUBA dives within the limits of the diver’s certification level, as recommended by the certifying agency.

  - SCUBA Diving is not authorized on or from AAFB property.  This restriction does not apply to SCUBA dive lessons taught at the base swimming pool or from commercially sourced boat excursions off the coast of AAFB.

# Boating Safety.

  - Prior to participating in recreational boating activities, boaters and supervisors shall together review and comply with the PACAF HRA Program boating checklist.

  - Prior to participating in recreational boating activities, boaters shall call commercial number 211 to check the daily weather, tides, and any potential water related cautions/warnings.

  - Prior to participating in recreational boating activities, all AAFB military and DoD civilian personnel will ensure they establish a “float plan” in the event boaters become capsized or stranded at sea.  “Float plans” should include the assistance of an onshore responsible person who will follow up on the boater’s whereabouts if the boater goes overdue on their scheduled return time.  Float plans at a minimum will include the route of intended travel and expected launch/docking times and locations.

  - Boaters are highly recommended to carry adequate supplies and safety equipment to ensure the safety of every person onboard (e.g. extra water, food, fuel, extra flares, mirror, a flashlight, and extra batteries).  Life jackets and other required safety equipment for boating is directed by United States Coast Guard regulations.

  - If operating a boat off shore, boaters must ensure it is equipped with a working radio. Although CB radios are popular, their power and range is limited to a distance of three miles.  The preferred radio to carry is the VHF-FM radio all U.S. Coast Guard units monitor. VHF- FM radios have a range of more than 50 miles.

  - For further Guam boating safety tips, please visit the U.S Coast Guard Sector Guam Water Safety website at http://www.uscg.mil/d14/sectguam/safety.asp.

# Jungle Safety.

  - Entry into the jungle is strongly discouraged unless engaged in an approved, designated activity such as a field trip, Guam Boonie Stompers sanctioned hike, or hike coordinated through Outdoor Recreation on a well-known and marked Guam trail.   Be sure to dress appropriately for the hike and bring along enough food, water, and emergency supplies in case of inclement weather or loss of direction.

  - Recreational rappelling on AAFB is prohibited.

# Motorized Vehicles.

  - Please refer to 36 WGI 31-218, Andersen AFB Motor Vehicle Traffic Supervision, for guidance on motor vehicle safety.

# Motorcycle, moped, Low Speed Vehicle (LSV), and All-Terrain Vehicle (ATV) Operations:

  - Please refer to 36 WGI 31-218, Andersen AFB Motor Vehicle Traffic Supervision, for guidance on LSV and ATV operations.

# Government Vehicle Other (GVO).

  - Per AFI 91-207, “the majority of GVOs are off-highway motorized vehicles such as specialty/special purpose, material handling, construction or tactical vehicles.” John Deere Gators, Club Cars, and Kawasaki Polaris vehicles are examples of GVOs.

  - Per AFI 91-207, paragraph 2.2.2.1., “Commanders will limit the use of GVOs to off-road areas and tactical operations as much as possible.” Prior to utilizing GVOs on AAFB, organizational commanders must ensure operating risk assessments are conducted and documented IAW AFI 90-802, for each type of GVO utilized by the organization.  Additionally, organizational commanders will ensure every GVO has a written plan of instruction (POI), IAW AFI 24-301 and AFMAN 24-306, to include identifying vehicle operational environment, usage requirements and manufacturer recommendations. Risk assessments, training plans, and operator certification records shall be developed, documented, and maintained by organizational Vehicle Control Officers or Unit Training Managers.

    - Traffic environments are defined as surfaces accessible to the base populous where government owned vehicles, privately owned vehicles, or contracted commercial vehicles may legally operate (streets, road ways, parking lots, etc).  Industrial environments are defined as surfaces not accessible to the base populous to include, but not limited to, unit training compounds, warehouses, storage areas, garages, mobile equipment facilities/areas, and property within the airfield perimeter fence.   Pedestrian environments are defined as surfaces accessible to the base populous where non-unit or unrelated personnel traverse by walking.

  - If all requirements listed in para. 19.2 are met, organizational commanders can approve the routine use of GVOs in traffic environments, industrial or pedestrian environments based on a risk assessment IAW AFI 90-802 with the following exceptions:

    - GVOs are authorized to operate in traffic, industrial or pedestrian environments if they are in compliance with Public Law No 99-570, Commercial Motor Vehicle Safety Act of 1986 and CFR 571.500 FMVSS, LSV, and meet or exceed the following requirements:

      - Headlamps.

      - Front and rear turn signal lamps.

      - Tail lamps.

      - Stop lamps.

      - Reflex reflectors:  one red on each side as far to the rear as practicable, and one red on the rear.

      - An exterior mirror mounted on the driver's side of the vehicle and either an exterior mirror mounted on the passenger's side of the vehicle or an interior mirror.

      - A parking brake.

      - A windshield that conforms to the Federal motor vehicle safety standard on glazing materials (49 CFR 571.205).

      - A VIN that conforms to the requirements of part 565 Vehicle Identification Number.

      - A Type 1 or Type 2 seat belt assembly conforming to 49 CFR 571.209, installed at each designated seating position.

      - Use of GVOs shall be restricted to non-inclement weather conditions and speed limits not exceeding 35 miles per hour.  GVOs may cross roadways with higher posted speed limits.

    - GVOs not in compliance with CFR 571.500 are authorized to operate in traffic or pedestrian environments only for mission essential activities which cannot be accomplished utilizing other methods (for example, accomplishing GVO refueling/maintenance services or GVO deployment processing).  These mission essential activities are restricted to daylight hours and non-inclement weather conditions.

    - GVOs not in compliance with CFR 571.500 are authorized to operate in industrial environments during daylight hours, periods of darkness, and inclement weather conditions in support of mission essential activities which cannot be accomplished utilizing other methods.  However, GVOs shall be equipped with seatbelts for each occupied seat, as well as a beacon light/flashers and reflector tape installed on the front, rear, and both sides of the conveyance to increase visibility.  Reflective tape must be a minimum of 2 inches wide and 3 inches in length.

# Safety Awards Program.

  - It is important to recognize individual and squadron efforts in ensuring the safety of our people and the protection of our resources.  All units are expected to actively participate in the safety awards nomination process.

  - Individual and Team Safety Awards of Distinction:  The semi-annual awards are presented to the individuals and teams who made the greatest contribution to mishap prevention on AAFB.  One individual is selected for the Occupational Safety Individual Safety Award of Distinction and one team is selected for the Occupational Safety Team Safety Award of Distinction.  Individual and team awards are also presented for the Weapons and Flight safety disciplines.  Criteria for the awards include safety awareness, professional knowledge and performance, and the identification and elimination of hazards.  Any military or DoD civilian member not assigned to full-time safety duty positions (1S0 AFSC) are eligible, to include USRs.

  - Twice a year, commanders, supervisors or any member of the 36th Wing or tenant unit may submit nominations for the awards.  Nominations shall be prepared using a narrative format.  Narrative format instructions and templates can be found on the 36th Wing Safety SharePoint site.  Nominations will be solicited through electronic mail the month prior to scheduled ESOHC meetings.  Nominations should detail reasons for the nomination and should include specific actions by the individual that qualify them for the award.  The significance of the achievement is more important than the quantity of material in the nomination. Award winners shall be selected by the 36th Wing Chief of Safety and staff and are asked to attend the following ESOHC for presentation of the award.

  - Commander’s Safety Award:  This annual award is presented to the individual who, by his/ her actions, made the most outstanding contribution to the prevention of a mishap, or reducing its harmful effects.  One winner shall be selected from each category (weapons, occupational, and flight safety) from the winners of the semi-annual awards.

  - Squadron Safety Award:  This award is presented annually to the squadron or unit who demonstrated the best overall safety program during the previous calendar year.  The award consists of a traveling trophy.  A recommendation shall be prepared by the 36th Wing Safety office, with the 36th Wing Commander selecting the eventual winner.  To qualify for this award the unit must not have experienced a class A mishap within the calendar year.  Annual and spot inspection findings from the year shall be reviewed.  Criteria for selection shall include:

    - Commander and supervisory emphasis on safety and RM.

    - Implementation of the AFOSH Program within the unit.

    - Mishap record relative to size of unit and degree of risk involved in normal activities.

    - Frequency and quality of unit safety meeting.

    - Effectiveness of USR.

    - Safety awareness and work practices of squadron personnel.

    - General conditions of work areas.

    - Timeliness and effectiveness of actions taken to correct safety deficiencies.

    - Quantity and quality of safety suggestions and Hazard Reports submitted.

DOUGLAS A. COX

Brigadier General, USAF

Commander

Attachment 1

GLOSSARY OF REFERENCES AND SUPPORTING INFORMATION

## References

AFPD 91-2, Safety Programs

AFI 90-801, Environment, Safety, and Occupational Health Council (ESOHC)

AFI 90-802, Risk Management

AFI 91-202, The US Air Force Mishap Prevention Program

AFI 91-203, Air Force Consolidated Occupational Safety Instruction

AFI 91-204, Safety Investigations and Report

AFI 91-207, The US Air Force Traffic Safety Program

DODI 6055.07, Mishap Notification, Investigation, Reporting, and Record Keeping

## Abbreviations and Acronyms

AAFB—Andersen Air Force Base

ADWSR—Additional Duty Weapon Safety Representative

AE—Ammunition and Explosives

AFOSH—Air Force Office of Safety and Health

ANSI—American National Standards Institute

ATV—All-Terrain Vehicle

CAT—Crisis Action Team

CES—Civil Engineering Squadron

EED—Electro-Explosive Device

EOC—Emergency Operation Center

ESOHC—Environmental, Safety, and Occupational Health Council

GVO—Government Vehicle Other

HRA—High Risk Activity

LRS—Logistics Readiness Squadron

LSV—Low Speed Vehicle

MSR—Motorcycle Safety Representative

MUSTT—Motorcycle Unit Safety Tracking Tool

OSHA—Occupational Safety and Health Administration

OSMB—Occupational Safety Management Book

PFD—Personal Floatation Device

PPE—Personal Protective Equipment

PT—Physical Training

RAC—Risk Assessment Code

RDS—Records Disposition Schedule RM—Risk Management

TCOR—Typhoon Conditions of Readiness

USR—Unit Safety Representative

UWSMB—Unit Weapons Safety Management Book

Attachment 2

OCCUPATIONAL SAFETY MANAGEMENT BOOK

A2.1. Unit Safety Representatives (USRs) will maintain an Occupational Safety Management Book (OSMB).  The OSMB will consist of:

A2.1.1. Tab A, Table of Contents.

A2.1.2. Tab B, Letter of Appointment and Record of Training.  Each unit will have a primary and alternate USR, as applicable.  When possible, these individuals will have one-year retainability. Training will be accomplished within 30 working days after appointment.  Appointment letters and the record of training will be maintained in this tab.

A2.1.3. Tab C, Mishap Notification Procedures.  USRs will assist commanders to develop and establish mishap notification procedures.

A2.1.4. Tab D, Workplace Identifier.  USRs will maintain a current listing of all facilities owned/used by their unit for inspection purposes.

A2.1.5. Tab E, Unit Annual Safety Inspection.  USRs will maintain the previous annual safety inspections to use for reference, providing follow-ups for open discrepancies to the wing safety office every 30 days until closed.

A2.1.6. Tab F, Unit Spot Inspections.  USRs will conduct and document spot inspections in conjunction with the facility manager when possible.

A2.1.7. Tab G, Hazard Reports and RAC 4/5 Log.  USRs will maintain information concerning hazards identified within their unit, such as AF Forms 457s, hazard reports, and safety inspection findings.

A2.1.8. Tab H, Mishap Reports.  USRs will maintain a copy of all AF Form 978s submitted to the wing safety office.  All reports will be submitted to the safety office within 5 calendar days of the mishap.  USRs will maintain a log of all AF Form 978s.

A2.1.9. Tab I, General Correspondence.  USRs will maintain special unit guidance provided by the commander, copies of any correspondence related to safety, i.e., special interest items, programs, publicity, and seasonal campaigns, and actions taken at the unit level.

A2.1.10. Tab J, Safety Briefings, Safety Information, Material Dissemination. USRs will maintain a copy of all safety briefings or information briefed to squadron personnel.  Additionally, maintain any safety materials provided by the Wing, PACAF safety, or outside agency.

A2.1.11. Tab K, Committee Meeting Minutes.  USRs will attend all USR meetings, and when invited, Environmental, Safety, and Occupational Health (ESOH) council meetings.  USRs will maintain meeting minutes for the previous year.

A2.1.12. Tab L, Special Programs Listing.  USRs will maintain a list of any special programs identified within the unit.  Maintain a list of Confined Spaces, Lock-Out/Tag-Out programs, Blood Borne Pathogens, etc.  Programs in this listing may be kept in separate binders.

A2.1.13. Tab M, Commanders Review Log.  USRs will meet with the Commander at least quarterly.  During this meeting, USRs will brief the CC on the health of the squadron’s overall mishap prevention program.  Discuss mishaps that occurred during the quarter, safety inspections and open findings, any safety training that is required, and any other high interest items for the quarter.

A2.1.14. Tab N, Miscellaneous.  Use this tab to maintain any additional information that does

not fall under the previous tabs.

A2.2. Organizations may augment the primary and alternate safety representatives using a “team concept” by adding representatives at the flight level (or equivalent organizational levels).  If the “Team Concept” is used, please refer to the 36 WG Occupational Safety page under Section Rep Toolbox for suggested shop/section/office safety management binder information.

A2.3. Unit Safety Representative (USR) Quarterly Report to the Unit CC Form.

A2.2.1. Use of the USR Quarterly Report to the Unit CC Form is highly recommend, and the form can be accessed via the AAFB SharePoint at https://andersen.eis.pacaf.af.mil/WingStaffAgencies/36WGSE/36WGSEG/_layouts/15/start.aspx#/Shared%20Documents/Forms/AllItems.aspx?RootFolder=%2FWingStaffAgencies%2F36WGSE%2F36WGSEG%2FShared%20Documents%2FUSR%20Toolbox%2FUSR%20Program%20Management%20Tabs%2FTab%20M%20%2D%20Commander%27s%20Review%20Log&FolderCTID=0x01200057790B338FD11E4086699E01BF06BC51&View=%7B915C5CCD%2D51C8%2D4EE0%2D98F7%2D6DDB246CB209%7D.

Figure A2.1.  The Unit Safety Representative (USR) Quarterly Report to the Unit CC Form

Attachment 3

UNIT WEAPONS SAFETY MANAGEMENT BOOK

A3.1. Each unit with an explosive safety program is required to maintain a UWSMB either hard copy or centrally-accessible electronic copy continuity binder.  This book is a management tool for each unit and the 36 WG/SEW to ensure continuity of the Weapons Safety Program.  The book shall contain, at a minimum, the following tabs:

A3.1.1. Tab A, Commanders’ Safety Emphasis Letters.

A3.1.2. Tab B, ADWSR appointment letter and documented training.

A3.1.3. Tab C, ADWSR guide and/or local publications.

A3.1.4. Tab D, Inspection checklists.

A3.1.5. Tab E, Unit spot inspection logs.

A3.1.6. Tab F, Past two annual unit inspection reports.

A3.1.7. Tab G, Weapons Safety Crosstell.

A3.1.8. Tab H, Copies of all unit AF Forms 2047, Explosive Safety License.

A3.1.9. Tab I, Miscellaneous Information.

# Attachment 4

USR PROGRAM ASSESSMENT CHECKLIST

Table A4.1.  USR Program Assessment Checklist

|  | Yes | No | N/A | Notes |
| --- | --- | --- | --- | --- |
| 1. Commanders/Functional Managers |  |  |  |  |
| 1.1. Does the commander provide safe and healthful workplaces throughout the unit? (AFI 91-202, 1.5.20.1.) |  |  |  |  |
| 1.2. Do all individuals receive the necessary job safety and off-duty safety training/briefings? (AFI 91-202, 1.5.20.2.) |  |  |  |  |
| 1.3. Does the commander support safety award programs? (AFI 36-2833, 2.2.) |  |  |  |  |
| 1.4. Does the commander attend safety councils and committees? (AFI 90-801, 4.3.) |  |  |  |  |
| 1.5. Does the unit track identified hazards/discrepancies until closure? (AFI 91-202, 1.5.20.5.) |  |  |  |  |
| 1.6. Has the Primary and Alternate USR with 12 months retainability been appointed in writing by the commander? (36 WGI 91-202, 2.1 and 2.2.) |  |  |  |  |
| 1.7. Did the primary and alternate attend USR training course within 30 working days of appointment? (AFI 91-202, 2.2.) |  |  |  |  |
| 1.8. Does USR conduct and document safety spot inspections at least monthly to ensure 100% unit coverage annually? (AFI 91-202, 8.5.6.) |  |  |  |  |
| 1.9. Does the USR attend scheduled USR meetings? (AFI 91-202, 8.5.8. and 36 WGI 91-202, 2.4.) |  |  |  |  |
| 1.10. Does the USR use the 36 WG/SEG Program Assessment Checklist to conduct safety self-inspections? (36 WGI 91-202, Attachment 4) |  |  |  |  |
| 1.11. Does the unit provide a response for Annual, Special and Spot inspection reports every 30 days until closure? (AFI 91-202, 3.5.1.) |  |  |  |  |

| 2. Occupational Safety Management Book (OSMB) |  |  |  |  |
| --- | --- | --- | --- | --- |
| 2.1. Does the USR maintain an OSMB IAW 36 WGI 91-202, Assessment Checklist? (36 WGI 91-202, Attachment 2) |  |  |  |  |
| Tab A: Table of Contents. |  |  |  |  |
| Tab B: Letter of Appointment and Record of Training. (AFI 91-202, 2.2.) |  |  |  |  |
| Tab C: Mishap Notification Procedures. Ensure mishap notification procedures are established in the unit. (AFI 91-202, 8.5.3.) |  |  |  |  |
| Tab D: Workplace Identifier. Provide the wing safety office with a current listing of all faciliites owned/used by the unit. (AFI 91-202, 8.5.8.) |  |  |  |  |
| Tab E: Unit Annual Safety Inspection. Maintain the last annual inspection report. (AFI 91-202, 3.6.1) |  |  |  |  |
| Tab F: Unit Spot Inspections. Documentation of USR monthly spot inspections. (AFI 91-202, 2.2.2.2., 3.7., & 8.5.5.) |  |  |  |  |
| Tab G: Hazard Reports and RAC 4/5 Log. Hazard Abatement Log with AF Form 3s. Maintain information concerning hazards identified within your unit (such as: AF Forms 457, Hazard reports, and Safety Inspection findings). These are hazards or open findings from the unit’s annual safety inspection. (AFI 91-202, 8.5.2.) |  |  |  |  |
| Tab H: Mishap Reports. Unit mishap log with AF Forms 978. (AFI 91-202, 2.2.2.3.) |  |  |  |  |
| Tab I: General Correspondence. The following are examples that should be maintained within this section: Special unit guidance provided by the commander, copies of correspondence related to occupational safety (i.e., special interest items, programs, publicity, and seasonal campaigns), and actions taken at unit level regarding correspondence. |  |  |  |  |
| Tab J: Safety Briefings, Safety Information, Material Dissemination. Documentation of organization/unit safety meetings/safety information/material dissemination. (AFI 91- 202, 8.5.4.) |  |  |  |  |
| Tab K: Committee Meeting Minutes. Maintain USR and ESOH Council meeting minutes for at least one year. (AFI 91-202, 8.5.6.) |  |  |  |  |
| Tab L: Special Programs Listing. Maintain listing of special programs found throughout the unit which apply to the work centers (Confined Spaces, Lock-Out/Tag-Out, and Blood Borne Pathogens, GMV Risk Assessments, etc). |  |  |  |  |
| Tab M: Commanders Review Log. Quarterly Commander’s Review of Safety Program (AFI 91-202, 8.5.1.) Utilize the USR Quarterly Report to the Unit Commander Form (36 WGI 91-202, Attachment A2.1.) |  |  |  |  |
| Tab N: Miscellaneous. This tab will be used for anything that was not mentioned previously. |  |  |  |  |

| 3. Bulletin Boards |  |  |  |  |
| --- | --- | --- | --- | --- |
| 3.1. AFVA 91-209, Air Force Occupational Safety and Health Program. |  |  |  |  |
| 3.2. Unit mishap reporting procedures. |  |  |  |  |
| 3.3. CA-10 if Civilian Employees work in the area. |  |  |  |  |
| 3.4. Are AF Form 457, USAF Hazard Report, and instructions for use, readily available to all personnel? (Posted on safety bulletin boards.) |  |  |  |  |
| 3.5. Current Mishap Prevention information (posters, flyers, safety grams, etc.). |  |  |  |  |
| 3.6. Wing Safety current POCs. |  |  |  |  |
| 3.7. BE Industrial Hygiene surveys posted for a minimum of 10 days. (AFI 91-202, 1.5.20.10.) |  |  |  |  |
| 3.8. OSHA Form 300A (1 Feb – 30 Apr annually) |  |  |  |  |
| 3.9. Wing Commander’s Safety Policy |  |  |  |  |
| 4. Safety Training (AF Form 55/JSTO/SST/HAZCOM) |  |  |  |  |
| 4.1. Do supervisors create and conduct job safety training outlines (JSTO) with all required items? Consider workcenter (local) HAZCOM training, LO/TO training, etc. (AFI 91-202, 1.5.21.4 and 2.3.3.) |  |  |  |  |
| 4.2. Have all personnel (military and civilian) received job safety training? (AFI 91-202, 2.3.3.1.) |  |  |  |  |
| 4.3. Do supervisors document the annual review of the workcenter JSTO? (AFI 91-202, 2.3.3.2.) |  |  |  |  |
| 4.4. Do all personnel (military and civilian) have job safety training documented on AF Form 55 or equivalent product? (AFI 91-202, 2.3.3.3.) |  |  |  |  |
| 4.5. Does the commander ensure personnel requiring supervisor safety training are scheduled and attend? (AFI 91-202, 2.3.2.) |  |  |  |  |
| 4.6. Does the unit document supervisor safety training on AF Form 55 or equivalent product? (AFI 91-202, 2.3.3.3.) |  |  |  |  |
| 4.7. Do supervisors know the safety and occupational health standards that apply to their areas and ensure standards are available? (AFI 91-202, 1.5.21.1.) |  |  |  |  |

| 5. Unit RM Program |  |  |  |  |
| --- | --- | --- | --- | --- |
| 5.1. Does the commander ensure RM is incorporated into job safety training outlines (JSTOs) and on-the-job (OJT) awareness training IAW AFI 91-202, and paragraphs 11.33. & 12.4 (AFI 90-802) |  |  |  |  |
| 5.2. Does the unit ensure all members have completed RM Fundamentals training? (AFI 90-802, para. 4.13.1.) |  |  |  |  |
| 6. Mishap Reporting |  |  |  |  |
| 6.1. Are written unit mishap reporting procedures established and used? (36 WGI 91-202, 6.1.) |  |  |  |  |
| 6.2. Are AF Form 978s signed by the commander sent to the Safety Office within 5 calendar days of the mishap occurrence? (36 WGI 91-202, 6.7.) |  |  |  |  |
| 7. Traffic Safety |  |  |  |  |
| 7.1. Have RM assessments been accomplished when government off-road vehicles are authorized to be used in traffic, industrial, or pedestrian environments? Owning organizations shall complete a written RM assessment on recreational type vehicles (i.e. Gators, off- road golf carts, agricultural carts, golf course maintenance tractors and carts). (AFI 91-207) |  |  |  |  |
| 7.2. Has a standard Plan of Instruction (POI), been written and training documented for specific vehicle types? (AFI 91-207, 4.7.3.) |  |  |  |  |
| 7.3. Has the commander approved the GMV for use, based off risk assessment? (36 WGI 91-202) |  |  |  |  |
| 7.4. Have operators of government owned ATVs completed the Specialty Vehicle Institute of America’s ATV Rider Course? (AFI 91-207, 4.4.7.3.) |  |  |  |  |

| 8. Motorcycle Program |  |  |  |  |
| --- | --- | --- | --- | --- |
| 8.1. Has unit commander appointed a MSR for the unit’s motorcycle safety program? (AFI 91-207, 1.3.3.14., 1.3.4.3.) |  |  |  |  |
| 8.2. Does the MSR maintain the unit motorcycle roster in MUSTT? (AFI 91-207, 3.5.6.) - Squadron In-Processing/Out-Processing Checklist Recommended for Unit MSR. |  |  |  |  |
| 8.3. Has the unit commander or designated MSR(s) provided an initial briefing to all new or newly assigned motorcycle riders within 30 days from initial assignment, identification as a rider or purchase of a motorcycle? (AFI 91-207, 3.5.7.) |  |  |  |  |
| 8.4. Has an annual/pre-season motorcycle riders’ briefing been performed by the unit commander or designated representative? (AFI 91-207, 3.5.8.) |  |  |  |  |
| 8.5. Is the MSR briefing personnel on motorcycle licensing, training, registration guidelines, etc? (AFI 91-207, 1.3.3.14.) |  |  |  |  |
| 8.4. Have all motorcycle operators attended an approved MSF course? (AFI 91-207, 4.5.1.) |  |  |  |  |
| 9. Personal Protective Equipment (PPE) |  |  |  |  |
| 9.1. Do commanders ensure a proactive mishap prevention program is implemented to include procurement and proper use of PPE, and facility compliance with AFI and OSHA standards. (AFI 91-202, 1.5.20.4.) |  |  |  |  |
| 9.2. Do supervisors exercise control over job tasks to ensure personnel follow all precautions and safety measures, including the proper use of PPE. (AFI 91-202, 1.5.21.6.) |  |  |  |  |
| 9.3. Do individuals use and maintain recommended and appropriate PPE for job tasks. Inspect and maintain PPE IAW TO, manufacturer’s instructions or Bioenvironmental Engineering guidance. (AFI 91-202, 1.5.22.5.) |  |  |  |  |
| 9.4. Do supervisors conduct and document hazard assessments in each workplace where their employees are performing duties? If PPE is required, do supervisors ensure that PPE is provided, used and maintained in a sanitary serviceable condition? (AFI 91-203, 14.3.2.) |  |  |  |  |

| 9.5. Are supervisors maintaining discipline with regard to personnel wearing properly fitted PPE? Do supervisors consult the installation Occupational Safety office and BE concerning the selection and use of PPE? (AFI 91-203, 14.3.2.1.) |  |  |  |  |
| --- | --- | --- | --- | --- |
| 9.6. Do supervisors contact BE when workplace operations change to schedule appropriate evaluation when new hazardous materials are introduced/processed, procedures are changed, or engineering controls are modified or added? (AFI 91 203, 14.3.2.2.) |  |  |  |  |
| 9.7. Do personnel ensure provided PPE is used when required, adjusted to properly fit and maintained in a sanitary and serviceable condition? (AFI 91-203, 14.3.3.3.) |  |  |  |  |
| 9.8. Does PPE meet minimum requirements? (Provide adequate protection, reasonably comfortable, fit snugly without interfering with movement, durable, capable of being disinfected, can easily be cleaned and kept clean and in good repair) (AFI 91-203, 14.4.2.1.) |  |  |  |  |
| 10. HAZCOM |  |  |  |  |
| - Is the written work-area specific HAZCOM program included and meet the following criteria IAW AFI 91-821, 3.1.? - Hazardous Chemical List - Non-Routine Tasks Involving Hazardous Chemicals – OI, specific task list, and job safety analyses thoroughly describe non-routine tasks, associated hazards and controls. - Labels and Other Forms of Warning – ensure labels on containers of hazardous chemicals used in their work area meet 29 CFR 1910.1200(f). - Safety Data Sheet – Work areas shall maintain a SDS (paper or electronic) for every item on the work area-specific hazardous chemical list. - Contractors in AF Work Areas – When an AF work area uses hazardous chemicals in a way that contractor employees may be exposed, then access to the work area-specific written HAZCOM program must be provided to the contractors’ area supervisor. - Employee Information and Training - Supervisors and employees who handle, use, or are potentially exposed to hazardous chemicals in the course of official AF duties must be provided HAZCOM training prior to the use of hazardous chemicals. (Supervisor Training, Initial Worker Training, |  |  |  |  |

| Supplemental Worker Training, Activities Not Co-Located and Worker Knowledge Assessment). |  |  |  |  |
| --- | --- | --- | --- | --- |
| 11. Eyewash |  |  |  |  |
| 11.1. Are permanently installed eyewash units conspicuously marked and accessible within 100 feet and no more than 10 seconds of reach to the exposure of hazardous materials? (AFI 91-203, 19.5 and 19.6.) |  |  |  |  |
| 11.2. Are energized circuits located within two (2) feet of emergency eyewash stations and five (5) of emergency shower stations protected by ground fault circuit interrupters (GFCI)? (AFI 91-203, 19.5.) |  |  |  |  |
| 11.3. Are permanently installed eyewash units inspected monthly? Does documentation include date of inspection and individual performing the inspection? (AFI 91-203, 19.9.2.) |  |  |  |  |
| - Are self-contained units tested and inspected IAW the manufacturer‘s instructions? - Is the fluid level checked monthly? - Are tags or labels attached to the unit or adjacent to it, indicating the fluid change schedule? (AFI 91-203, 19.9.3.) |  |  |  |  |
| 12. Electrical Work Areas |  |  |  |  |
| 12.1. Do all Aircraft Maintenance personnel receive CPR training at least every two years? (AFI 91-203, 24.2.3.1.) |  |  |  |  |
| 12.2. Do all electrical workers (military and civilian) receive CPR training annually? (AFI 32-1064, 2.13.2.) |  |  |  |  |
| 12.3. Is emergency safety equipment available and situated in a conspicuous location, well-marked, and readily accessible by all personnel? (AFI 91-203, 8.12.) |  |  |  |  |
| 13. Hangar Doors Program |  |  |  |  |
| 13.1. Are only qualified personnel approved by the commander in writing authorized to operate the hangar doors? (AFI 91-203, 24.14.8.3.) |  |  |  |  |
| 13.2. Are written operating procedures outlining safety precautions published and posted next to operating controls? (AFI 91-203, 24.14.8.3.) |  |  |  |  |
| 13.3. Is a hinged cover or other device installed over the close switch to prevent inadvertent activation? (AFI 91-203, 24.14.8.2.) |  |  |  |  |
| 13.4. Are powered hangar doors equipped with alarm sounding devices with a sound that is distinguishable from the facility fire alarm and audible above normal noise levels? (AFI 91-203, 24.14.8.1.) |  |  |  |  |

| 13.5. Does the alarm sounding device automatically signal at least 5 seconds before any door section movement and sound continuously while the door is being operated and resets immediately after movement stops? (AFI 91-203, 24.14.8.1.) |  |  |  |  |
| --- | --- | --- | --- | --- |
| 13.6. Are luminescent or reflective directional arrows placed adjacent to each switch and on horizontal sliding doors to indicate the direction of door travel for each corresponding switch? (AFI 91-203, 24.14.8.2.2.) |  |  |  |  |
| 14. Ladder Safety Program |  |  |  |  |
| 14.1. Portable Ladders: |  |  |  |  |
| 14.1.2. Are personnel who use ladders at four feet and higher trained in the care and use of different types of ladders when first assigned? Information presented during safety briefings shall satisfy the periodic training requirement. All training shall include hands-on instruction to include inspection of ladders for defects, possible electrocution hazards, proper positioning, and placement of ladders for various job sites. Training shall be documented IAW AFI 91-202 and AFI 91-203, 7.5.2.7. |  |  |  |  |
| 14.1.3. Are ladders with conductive side marked for electrical hazards? Ladders not already marked with safety use instructions by the manufacturer shall be stenciled, ―DANGER — DO NOT USE AROUND ELECTRICAL EQUIPMENT, in two-inch high red letters or the largest letters the surface shall allow (minimum letter size is one inch). (AFI 91-203, 7.5.2.2.6.) |  |  |  |  |
| 14.2. Fixed Ladders: |  |  |  |  |
| 14.2.1. Is a formal, fully documented inspection of all fixed ladders accomplished least every 3 years? Inspection documentation shall include the type ladder and safety climb device installed, location, a detailed list of all discrepancies, and corrective action status. (CES maintains documentation; however a copy of inspection documentation should be maintained at unit level). (AFI 91-203, 7.4.3.1.) |  |  |  |  |
| 14.2.2. Does the inspection documentation include the type of ladder and safety climb device installed, location, a detailed list of all discrepancies and corrective action status? (AFI 91-203, 7.4.3.1.) |  |  |  |  |
| 15. Forklift Program |  |  |  |  |
| 15.1. Do workcenters requiring forklift training designate qualified personnel in writing as instructors? |  |  |  |  |

| (AFI 91-203, 35.3.3.1.) |  |  |  |  |
| --- | --- | --- | --- | --- |
| 15.2. Does materials handling and lifting equipment training include references 35.3.3.4.1- 35.3.3.4.15.? (AFI 91-203, 35.3.3.4.) |  |  |  |  |
| 15.3. Does the instructor provide initial training and certification to personnel who operate powered forklifts on the AF Form 171, Request for Driver’s Training and Addition to U. S. Government Driver’s License? (AFI 91-203, 35.3.3.3.) |  |  |  |  |
| 15.4. Does a qualified instructor evaluate each operator at least once every three years and provide refresher training. (AFI 91-203, 35.3.3.6.) |  |  |  |  |
| 15.5. Are all powered industrial trucks inspected daily or prior to use by the operator and documented on AF Form 1800, Operator’s Inspection Guide and Trouble Report, for industrial tractors and tugs and AF Form 1800/1810, Operator’s Inspection Guide and Trouble Report, for forklifts? (AFI 91-203, 35.3.6.) |  |  |  |  |
| 15.6. Indoor ventilation: Has BE personnel conducted exhaust testing to preclude exposure to carbon monoxide when internal combustion engine equipment is used inside warehouses? (AFI 91-203, 35.3.5.1.) |  |  |  |  |
| 16. Hoist/Crane Program |  |  |  |  |
| 16.1. Do annual hoist/crane inspections include: (AFI 91-203) |  |  |  |  |
| 16.2. Complete inspection of all wire rope, chain, and fittings or attachments. (AFI 91-203, 35.5.2.3.2.1.) |  |  |  |  |
| 16.3. Inspection of brakes, pawls, or other holding features. (AFI 91-203, 35.5.2.3.2.2.) |  |  |  |  |
| 16.4. Inspection of the chain length. Chains that have elongated more than one-fourth of an inch in 12 inches shall be removed from service. (AFI 91-203, 35.5.2.3.2.3.) |  |  |  |  |
| 16.5. Are hoist identification tags attached to all hoists/cranes? (AFI 91-203, 35.5.2.3.3.) Tags shall include: |  |  |  |  |
| 16.6. Date of annual inspection (AFI 91-203, 35.5.2.3.3.1.) |  |  |  |  |
| 16.7. Date of load test (AFI 91-203, 35.5.2.3.3.2.) |  |  |  |  |
| 16.8. Capacity of hoist (AFI 91-203, 35.5.2.3.3.3.) |  |  |  |  |
| 16.9. Identification number of hoist (AFI 91-203, 35.5.2.3.3.4.) |  |  |  |  |
| 16.10. Are hoists operated only by personnel trained and qualified to use the equipment? (AFI 91-203, 35.6.3.3.) |  |  |  |  |
| 16.11. Are cranes operated only by properly trained and |  |  |  |  |

| qualified personnel? (AFI 91-203, 35.9.2.2.) |  |  |  |  |
| --- | --- | --- | --- | --- |
| - Crane and Powered Hoist: Is a visual inspection performed by the operator or designated person daily or prior to use? Is the inspection documented on an Air Force Technical Order (AFTO) Form 244, Industrial/Support Equipment Record, annotating AFI 91-203, 35.1.5.1.? - The inspection date. - The inspection time. - Initials of the person performing the inspection. - Any discrepancies noted during the inspection. |  |  |  |  |
| 17. Fall Protection Program {General Industry/Construction/Aircraft Flightline Operations} |  |  |  |  |
| 17.1. General Industry. Passive fall protection shall be provided whenever workers can fall four (4) feet or more. This four (4) foot rule applies to all walking and working surfaces and includes open-sided floors and platforms, wall openings and window wall openings at a stairway landing, floor, platform or balcony with a drop of four (4) feet or more. (AFI 91-203, 13.2.2.) |  |  |  |  |
| 17.2. Construction Operations. Fall protection shall be provided when workers can fall six (6) feet or more. This six (6) foot rule applies to all walking and working surfaces, including scaffolding, roofs, open-sided floors and platforms, wall openings and window wall openings at a stairway landing, floor, platform or balcony with a drop of six (6) feet or more. (AFI 91-203, 13.1.4.) |  |  |  |  |
| 17.3. Does the squadron have competent person, this means a person who can identify hazardous or dangerous conditions in personal fall arrest systems (PFAS) or any component thereof, and their applications and uses with related fall protection equipment? (AFI 91-203, 13.1.3.) |  |  |  |  |
| 17.4. Are supervisors ensuring initial and recurring training and training evaluations are provided to shop personnel on fall protection/fall arrest systems prior to initial use? (AFI 91-203, 13.5.1.3.) |  |  |  |  |
| - Does Fall Protection Training include? (AFI 91-203, 13.6.) - Application limits - Proper anchoring and tie-off techniques - Estimation of free fall distance - Determination of deceleration distance - Total fall distance to prevent striking a lower level - Methods of use |  |  |  |  |

| - Inspection - Storage - Manufacturer’s recommendations |  |  |  |  |
| --- | --- | --- | --- | --- |
| 17.6. Is Fall Protection training documented on the AF Form 623/55 or equivalent? (AFI 91-203, 13.6.) |  |  |  |  |
| 17.7. Are personnel using their fall protection equipment correctly? (AFI 91-203, 13.5.) |  |  |  |  |
| 17.8. Do supervisors ensure all PFAS components receive a thorough inspection at least quarterly? This inspection shall be documented and maintained for at least one year. (AFI 91-203, 13.4.6.10.3.) |  |  |  |  |
| 17.9. Has a self-inspection checklist been prepared and accomplished annually IAW TO 00-25-245. Checklist shall include all relevant information on fall protection/fall arrest systems, i.e., proper care, maintaining and inspection of fall protection/fall arrest systems equipment, training program, etc. The self- inspection shall be documented and maintained until the next self-inspection is performed. (AFI 91-203, 13.5.1.5.) |  |  |  |  |
| 17.10. Is the in service date of fall protection equipment documented? The service life of fall protection equipment manufactured of synthetic fiber shall be five (5) years (unless otherwise specified by the manufacturer). (AFI 91-203, 13.4.6.2.1.) |  |  |  |  |
| 17.11. Aircraft Flightline Operations, If procedural controls are used: local procedural controls, driven by conditions unique to a local facility or equipment design, or local operational requirements, shall be developed and documented in a JSA IAW AFI 91-202 by the maintenance group (or equivalent), approved by the maintenance group commander and coordinated with the organizational/appropriate Occupational Safety office before the procedural controls are used. (AFI 91-203, 24.16.5.3.) |  |  |  |  |
| 18. Hearing Protection Program |  |  |  |  |
| 18.1. Does the unit ensure workplaces and Workplace Supervisors comply with all AF Hearing Conservation Program (HCP), DoD, and OSHA requirements? (AFI 48-127, 2.11. & 2.12.1.) |  |  |  |  |
| 18.2. Is hearing conservation program briefed in workcenter job safety training? (AFI 91-202, A5.2.5.) |  |  |  |  |
| 19. Respiratory Protection Program |  |  |  |  |
| 19.1. Does the unit have workcenters assigned to the respiratory protection program? (AFI 48-137, 3.1.1.) |  |  |  |  |
| 19.2. Is respiratory protection program briefed in workcenter job safety training? (AFI 91-202, A5.2.8.) |  |  |  |  |
| 19.3. Do employees receive initial and periodic fit |  |  |  |  |

| testing (annual, and as changes occur) for industrial respiratory protection? (AFI 48-137, 2.21.7.) |  |  |  |  |
| --- | --- | --- | --- | --- |
| 20. Lockout/Tagout Program (LOTO) |  |  |  |  |
| 20.1. Does unit have workcenters that requires a LO/TO program? If No, skip to next section of this checklist. (AFI 91-203, 21.2.1.) |  |  |  |  |
| 20.2. Is LO/TO awareness training defined in the workcenter job safety training for all assigned personnel within the workcenter (affected employees)? (AFI 91-203 21.5.1., 21.5.1.1., and 21.5.1.2.) |  |  |  |  |
| 20.3. Is LO/TO training documented? All training, i.e., familiarization, initial and recurring, shall be documented. Training documentation shall be certified, current, include each individual‘s name, type and dates of training and be documented on the AF IMT 55, Employee Safety and Health Record, or equivalent product IAW AFI 91-202. All training documentation shall be readily available during inspections and LOTO program evaluations? (AFI 91-203, 21.5.5.) |  |  |  |  |
| 20.4. Does the work center maintain the following? |  |  |  |  |
| 20.4.1. LO/TO Authorization Letter. (AFI 91-203, 21.3.5.4.) |  |  |  |  |
| 20.4.2. Equipment Identification Survey. (AFI 91-203, 21.5.2.2.) |  |  |  |  |
| 20.4.3. LO/TO Procedures-must include 9 steps. (AFI 91-203, 21.4.9.2.) |  |  |  |  |
| 20.4.4. LO/TO Control Log. (AFI 91-203, 21.4.9.2.) |  |  |  |  |
| 20.4.5. LO/TO Inventory and Location of Locks. (AFI 91-203, 21.3.5.8.) |  |  |  |  |
| - Program Evaluation to include: - Shop Semi-Annual Inspection Date. (AFI 91-203, 21.6.) - Annual Wing Safety Inspection. (AFI 91-203, 21.6.) |  |  |  |  |
| 20.4.7. LO/TO Training and Documentation. (AFI 91-203, 21.5.2-21.5.5.) |  |  |  |  |
| 20.5. Does LO/TO training address that the authorized employee must have knowledge of the type and magnitude of the energy source, hazards of the energy source to be controlled and the method or means to control the energy? (AFI 91-203, 21.5.2.1.) |  |  |  |  |
| 20.6. Are procedures developed and documented for the safe and proper use of LO/TO devices? |  |  |  |  |

| (AFI 91-203, 21.2.1.4.) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| 20.7. Do authorized employees validate machine or equipment de-energized or isolated? (AFI 91-203, 21.4.2.6.3.) |  |  |  |  |  |
| 20.8. Once system is locked and tagged out, is all potentially stored or residual energy relieved, disconnected, or restrained? (AFI 91-203, 21.4.2.6.) |  |  |  |  |  |
| 20.9. Do procedures ensure the machine or equipment is clear of all items before the removal of lockout devices and restoration of energy to equipment or machine? (AFI 91-203, 21.4.2.8.2.2.) |  |  |  |  |  |
| 20.10. Have machines and equipment been identified on which the LO/TO program is used? (AFI 91-203, 21.3.9.2—listed in Tab 2 Equipment Survey) |  |  |  |  |  |
| 20.11. Are affected employees notified before LO/TO is applied and again upon removal of LO/TO within the workcenter? (AFI 91-203, 21.4.2.2., 21.4.2.9.2.—listed in Tab 3 Procedures.) |  |  |  |  |  |
| 20.12. Are specific procedures developed and utilized during shift or personnel changes to ensure the continuity of LO/TO protection, (orderly transfer of these devices between off going and on coming supervisors)? (AFI 91-203, 21.4.10.) |  |  |  |  |  |
| 20.13. Is an adequate supply of safety tags and locks available for use? (AFI 91-203 para 21.3.3.) |  |  |  |  |  |
| 20.14. Are shop LO/TO self-inspections conducted and documented at least semi-annually by the unit? (AFI 91-203, 21.6.1.) |  |  |  |  |  |
| 20.15. Are functional managers and commanders ensuring inspections of LOTO procedures within their organizations are conducted by an individual above the shop level to verify and document effectiveness of energy control procedures? (AFI 91-203, 21.6.2.) |  |  |  |  |  |
| 20.16. Is an inspection of the LO/TO program conducted at least annually by Occupational Safety? (AFI 91-203, 21.6.2.) |  |  |  |  |  |
| 21. Confined Space Program (CS) |  |  |  |  |  |
| 21.1. Does the unit maintain a complete inventory of all confined spaces in unit facilities or confined spaces unit personnel may enter? (AFI 91-203, 23.2.7.4.) |  |  |  |  |  |
| 21.2. Has the unit provided a copy of the list of all confined spaces, permit-required and non-permit, to the installation CSPT and Environmental Management office? The list shall include, at a minimum, the number of spaces (quantity), type (with every entry point uniquely identified |  |  |  |  |  |

| and listed) and exact location (e.g., grid coordinates, GPS coordinates, if available, highlighted maps, if necessary). (AFI 91-203, 23.2.7.5.) |  |  |  |  |
| --- | --- | --- | --- | --- |
| 21.3. Does the unit have permit required confined spaces in their facilities? If so: |  |  |  |  |
| 21.4. Are permit-required confined spaces labeled with a sign: “DANGER —PERMIT-REQUIRED CONFINED SPACE, DO NOT ENTER” and/or secured to prevent accidental entry where workers or other employees could inadvertently enter, and are all workers briefed to stay out? (AFI 91-203, 23.3.5.) |  |  |  |  |
| 21.5. Does the unit maintain initial testing and evaluation of each confined space (confined space assessments)? (AFI 91-203, 23.3.3.) |  |  |  |  |
| 21.6. Do unit personnel enter permit-required confined spaces? If N/A, skip to next section of this checklist. (AFI 91-203, 23.2.2.5.2. & 23.2.7.2.) |  |  |  |  |
| 21.7. Does the unit document all confined space training on the AF Form 55 or equivalent? (AFI 91-203, 23.7.5.) |  |  |  |  |
| 21.8. Do all confined space rescue team members receive annual training? (CEF & MXS Fuel Cell) (AFI 91-203, 23.7.3.1.) |  |  |  |  |
| 21.9. Does the workcenter have a confined space entry plan (MEP) approved by Safety, Fire, and BE annually? (AFI 91-203, 23.5.4.10.) |  |  |  |  |
| 21.10. Does the workcenter have a confined space lesson plan approved by Safety, Fire, and BE annually? (AFI 91-203, 23.7.1.1.) |  |  |  |  |
| 21.11. Does the MEP? (AFI 91-203, 23.2.5.4.) |  |  |  |  |
| 21.11.1. Describe the acceptable entry conditions, including acceptable atmospheric conditions, under which permits may be issued. (AFI 91-203, 23.2.5.4.1.) |  |  |  |  |
| 21.11.2. Designate entry supervisors. (AFI 91-203, 23.2.5.4.2.) |  |  |  |  |
| 21.11.3. Identify types and locations of spaces to be entered and types of tasks or operations to be performed. (AFI 91-203, 23.2.5.4.3.) |  |  |  |  |
| 21.11.4. List either by reference or direct statement in the MEP the procedures to be used for entry, e.g., shop OI that cover specific tasks. (AFI 91-203, 23.2.5.4.4.) |  |  |  |  |
| 21.11.5. Account for around-the-clock operations, when appropriate. (AFI 91-203, 23.2.5.4.5.) |  |  |  |  |
| 21.11.6. List PPE, monitoring and rescue equipment, and conditions under which it shall be used. (AFI 91-203, 23.2.5.4.6.) |  |  |  |  |

| 21.11.7. Designate frequency and type of atmospheric monitoring. (AFI 91-203, 23.2.5.4.7.) |  |  |  |  |
| --- | --- | --- | --- | --- |
| 21.11.8. List other controls required, e.g., lockout and/or tagout, ventilation. (AFI 91-203, 23.2.5.4.8.) |  |  |  |  |
| 21.11.9. List chemicals and quantities authorized for use. List expected exposure levels based on air sampling results. Based on exposure levels, perform reassessments of the confined spaces IAW requirements in paragraph 23.2.5.4.7. (AFI 91-203, 23.2.5.4.9.) |  |  |  |  |
| 21.11.10. List conditions under which the space may be reclassified as described in paragraph 23.5.4.11. (AFI 91-203, 23.2.5.4.10.) |  |  |  |  |
| 21.11.11. Provide procedures for amending the MEP. (AFI 91-203, 23.2.5.4.11.) |  |  |  |  |
| 21.11.12. Require verification of the condition of all monitoring equipment and PPE. (AFI 91-203, 23.2.5.4.12.) |  |  |  |  |
| 21.11.13. Be maintained by the entry supervisor at the worksite. (AFI 91-203, 23.2.5.4.13.) |  |  |  |  |
| 21.11.14. Include provisions for entry during potential emergency situations. (AFI 91-203, 23.5.1.2.) |  |  |  |  |
| 21.11.15. Establish emergency rescue procedures for each permit-required confined space. Include provisions for entry during potential emergency situations to include IDLH conditions. Refer to paragraph 23.6 for additional guidance. (AFI 91-203, 23.2.5.4.15.) |  |  |  |  |
| 21.11.16. Establish communication procedures and identify communication equipment to be used during entries. (AFI 91-203, 23.2.5.4.16.) |  |  |  |  |
| 21.11.17. Require the ready availability of rescue, atmospheric testing and safety-related equipment such as lifting or retrieval devices, respiratory equipment and other equipment, as necessary, for the entry as determined by the permit system. (AFI 91-203, 23.2.5.4.17.) |  |  |  |  |
| 21.11.18. Require adequate attachment points outside the confined space for tying-off or otherwise securing retrieval lines for all authorized entrants. (AFI 91-203, 23.2.5.4.18.) |  |  |  |  |
| 21.11.19. Require an equivalent method for rescue when retrieval lines themselves may constitute an entanglement hazard or otherwise cannot be used. 9AFI 91-203, 23.2.5.4.19.) |  |  |  |  |
| 21.11.20. Require availability of a rescue team. If the installation FES Flight is not available, verify availability of an organizational rescue team or other emergency |  |  |  |  |

| rescue team. The operation shall be halted if the primary rescue team becomes unavailable until the primary team returns or a secondary trained and qualified team is available. (AFI 91-203, 23.2.5.4.20.) |  |  |  |  |
| --- | --- | --- | --- | --- |
| 21.11.21. Require a reliable method, i.e., telephone, radio, etc., for summoning the rescue team and ensure it is operable, on hand or easily accessible. (AFI 91-203, 23.2.5.4.21.) |  |  |  |  |
| 21.11.22. Require functional manager or shop supervisor brief entry supervisor(s) on their duties prior to performing the operation. (AFI 91-203, 23.2.5.4.22.) |  |  |  |  |
| 21.12. Does the supervisor issue entry permits consistent with the MEP. (AFI 91-203, 23.2.8.1.1.) |  |  |  |  |
| - Are AF Form 1024s completed IAW approved MEP: - 1024 Section 7: Preparation documented IAW MEP - 1024 Section 8: Atmospheric readings (initial and intervals) documented IAW MEP Section 7 documented intervals - Supervisor signatures (space open time and space closed time) (AFI 91-203, 23.10.2.5; 23.10.2.6.) |  |  |  |  |
| 21.14. Are AF Form 1024s issued maintained on file for one year? (AFI 91-203, Figure 23.2, AF Form 1024) |  |  |  |  |
| 21.15. Does the unit provide the installation Occupational Safety office an annual status report on the installation‘s confined space program. The report shall include, as a minimum, the number and type of confined spaces, the installation‘s major issues/challenges, the number of personnel trained on confined space operations and the number of entry permits and MEPs approved. (AFI 91-203, 23.2.5.13.) |  |  |  |  |
| 21.16. Does the workcenter contact SEG/Fire/BE for non-routine entry permits? (AFI 91-203, 23.6.1.) |  |  |  |  |

# Attachment 5

ADWSR PROGRAM CHECKLIST

Table A5.1.  ADWSR Program Checklist

|  | Yes | No | N/A | Notes |
| --- | --- | --- | --- | --- |
| 1. ADWSR Management |  |  |  |  |
| NOTE: Chemical munitions requirements are identical to explosive requirements. |  |  |  |  |
| 1.1. Have all personnel (supervisory and non- supervisory) who operate, handle, transport, maintain, load, or dispose of missiles or explosives received initial/annual weapons safety training before performing any tasks? (AFI 91-202) |  |  |  |  |
| 1.2. Are procedures established and published for mishap notification? (AFI 91-202) |  |  |  |  |
| 1.3. Have unit lesson plans been developed to conduct initial and recurring training? (AFI 91-202) |  |  |  |  |
| 1.4. Are locally developed lesson plans coordinated through Wing Safety? (PACAFI 91-202) |  |  |  |  |
| 1.5. Is training conducted by unit weapons/explosives representative(s) or qualified instructor(s)? (PACAFI 91-202) |  |  |  |  |
| 1.6. Does the unit commander develop and implement safety program elements in his/her unit? (AFI 91-202) |  |  |  |  |
| 1.7. Is supervision aware and do they recognize deserving personnel or units by supporting the 36th Wing Mishap Prevention, 11th AF, and the PACAF Annual Safety Awards Program? (AFI 36-2833) |  |  |  |  |
| 1.8. If heat producing devices are required that produce temperatures higher than 228 degrees Fahrenheit, are written instructions developed and coordinated with weapons safety and the fire department for approval? (AFMAN 91-201) |  |  |  |  |
| 1.9. Are supervisors knowledgeable of hazards involved in the operation and do they convey emergency procedures to workers? (AFMAN 91-201) |  |  |  |  |
| 1.10. Are personnel technically trained and qualified to perform the task being conducted? (AFMAN 91-201) |  |  |  |  |
| 1.11. Is the cardinal principal of explosive safety (expose the minimum number of people to the minimum amount of explosives for the minimum amount of time) observed? (AFMAN 91-201) |  |  |  |  |
| 1.12. Does the unit ensure that the fire department pre- fire plans reflect current hazards? (AFMAN 91-201) |  |  |  |  |

| 1.13. Are written operating instructions for explosive operations available and approved by the squadron commander or equivalent? (AFMAN 91-201) |  |  |  |  |
| --- | --- | --- | --- | --- |
| 1.14. Do written operating instructions contain the minimum required items? (AFMAN 91-201) |  |  |  |  |
| 1.15. Are unserviceable/expended items turned in to the base munitions storage area as quickly as possible to preclude build-up of unserviceable NEWQD? Unserviceable NEWQD must be counted against the total NEWQD of the licensed facility. (AFMAN 91-201) |  |  |  |  |
| 1.16. Is the fire department notified when an explosive fire, chemical hazard and/or fire direction symbol is changed or added to a facility? (AFMAN 91-201) |  |  |  |  |
| 1.17. Is flammable storage located at least 50 feet from explosive locations? (AFMAN 91-201) |  |  |  |  |
| 1.18. Is good housekeeping apparent in explosive storage/licensed facilities and operating locations? (AFMAN 91-201) |  |  |  |  |
| 1.19. Are storage/licensed locations maintained in good repair and suitable for the types and hazards involved? (AFMAN 91-201) |  |  |  |  |
| 2. ADWSR Responsibilities |  |  |  |  |
| 2.1. Does the ADWSR ensure that operating instructions, lesson plans, and explosive licenses are reviewed and coordinated with the proper agencies and are they reviewed during the annual inspection/assessment? (AFI 91-202, WG. Sup 1) |  |  |  |  |
| 2.2. Does the ADWSR perform and document monthly spot inspections on all explosives activities in their unit? (AFI 91-202, WG Sup 1) |  |  |  |  |
| a. Are all shifts included? |  |  |  |  |
| b. Has the commander published inspection frequency guidance? |  |  |  |  |
| c. Are spot inspection discrepancies monitored until corrected? |  |  |  |  |
| 2.3. Does the ADWSR monitor compliance with safety directives, safety training and use of qualified personnel/equipment within their unit? (AFI 91-202, WG Sup 1) |  |  |  |  |
| 2.4. Does the ADWSR distribute safety data and post information to bulletin boards? (AFI 91-202, WG Sup 1) |  |  |  |  |
| 2.5. Has the ADWSR been appointed in the organizations |  |  |  |  |

| that handle, maintain, store, install or remove missiles, explosives, chemicals or nuclear weapons? (PACAFI 91-202) |  |  |  |  |
| --- | --- | --- | --- | --- |
| 2.6. Does the ADWSR inform the commander of unit specific weapons safety issues and weapons safety program? (AFI 91-202, WG SUP 1) |  |  |  |  |
| 2.7. Has the newly appointed ADWSR attended training within 30 days of being appointed? (AFI 91-202) |  |  |  |  |
| - Do ADWSRs receive recurring training 12 months after initial training? (AFI 91-202 PACAF SUP) |  |  |  |  |
| 2.8. Does the ADWSR maintain a weapons safety continuity book? (AFI 91-202, WG Sup 1) |  |  |  |  |
| 2.9. Has the ADWSR established and published written internal explosive mishap reporting procedures? (AFI 91-202, WG Sup 1) |  |  |  |  |
| 3. ADWSR Explosive Facility Licenses |  |  |  |  |
| 3.1. Are facilities that are not sited and store small amounts of ammunition and explosives (AE) licensed in accordance with Chapter 11? (AFMAN 91-201) |  |  |  |  |
| 3.2. Are explosive licenses reviewed annually? (AFMAN 91-201) |  |  |  |  |
| 3.3. Are explosive licenses coordinated through responsible Munitions Accountable System Officer, the local Security Forces Resource Protection office and the base fire protection agency prior to being approved by the installation Weapons Safety Office? (AFMAN 91-201) |  |  |  |  |
| 3.4. Are fire extinguisher(s) type, quantity, and placement as designated by the Fire Department included on the license? (AFMAN 91-201) |  |  |  |  |
| 3.5. Are explosive licenses clearly displayed at the location and current? (AFMAN 91-201) |  |  |  |  |
| a. Do licenses clearly state location of explosives authorized? (AFMAN 91-201) |  |  |  |  |
| b. Are stored explosive quantities within licensed authorization? (AFMAN 91-201) |  |  |  |  |
| c. Are licenses updated each time the HD, NEWQD, CG, or quantity of AE items change? (AFMAN 91-201) |  |  |  |  |
| 3.6. Has a local OI been produced to cover storage, issue, transporting, and assembly procedures of explosives? (AFMAN 91-201) |  |  |  |  |
| a. Non-explosives Waste Materials. (AFMAN 91-201) |  |  |  |  |
| b. Explosives Residue. (AFMAN 91-201) |  |  |  |  |
| c. Cleaning Compounds. (AFMAN 91-201) |  |  |  |  |

| 3.7. Are explosives operating and personnel limits posted at appropriate explosive locations? (AFMAN 91-201) |  |  |  |  |
| --- | --- | --- | --- | --- |
| 3.8. Are appropriate fire and chemical hazard symbols posted on and visible from all approach roads? (AFMAN 91-201) |  |  |  |  |
| 3.9. Are fire extinguishers readily available at the facility? (AFMAN 91-201) |  |  |  |  |
| 3.10. Is the structure/facility used for storage locked and secured to prevent pilferage and unauthorized handling? (AFMAM 91-201) |  |  |  |  |
| 3.11. Are outer containers in good condition and securely closed? (AFMAN 91-201) |  |  |  |  |
| 3.12. Are unserviceable explosive items segregated and properly marked? (AFMAN 91-201) |  |  |  |  |
| 3.13. Are training items physically separated from the live items they represent? (AFMAN 91-201) |  |  |  |  |
| 3.14. Does the stored quantity exceed the level authorized by the license and the smallest quantity needed? (AFMAN 91-201) |  |  |  |  |
| 3.15. Are all parts of explosives stacks provided ventilation by the use of dunnage? (AFMAN 91-201) |  |  |  |  |
| 3.16. Are the stocks provided dunnage for ventilation when required by civil engineering, logistics or bioenvironmental directives? (AFMAN 91-201) |  |  |  |  |
| 3.17. Are static grounds for equipment inspected? (AFI 32-1065) |  |  |  |  |
| 3.18. Are suitable fragmentation barriers used for class/division 1.2.2 or (04) 1.2 explosives? (AFMAN 91-201) |  |  |  |  |
| 3.19. Is smoking allowed in an explosives storage area or operating location only in specifically designated locations, where “authorized smoking area” signs are posted? (AFMAN 91-201) |  |  |  |  |
| 3.20. When unpackaged EED items are stored in the licensed facility, are grounding points available for personnel to discharge static electricity prior to handling explosives? (AFMAN 91-201) |  |  |  |  |
| 3.21. Are munitions properly closed and clearly marked to show contents and quantity? (AFMAN 91-201 / TO 11A-1-10 / Specific Item TO) |  |  |  |  |

| 4. ADWSR Transportation |  |  |  |  |
| --- | --- | --- | --- | --- |
| 4.1. Are drivers qualified to operate the vehicle and knowledgeable of the explosives being transported and associated hazards? In addition, do civilian drivers have Commercial Drivers Licenses, with a hazardous materials endorsement, to transport explosives off a military installation? (AFI 24-301) |  |  |  |  |
| 4.2. Are drivers aware of and comply with all transportation requirements? (AFMAN 91-201) |  |  |  |  |
| 4.3. Are explosive laden vehicles equipped with at least two serviceable, portable fire extinguishers rated at a minimum of 2A:10BC? In addition, is one mounted on the exterior driver’s side and one mounted inside the cab? (AFMAN 91-201) |  |  |  |  |
| 4.4. Are explosives placarding requirements being met? (AFMAN 91-201, 36 WG SUP) |  |  |  |  |
| 4.5. Has a LOI been approved to transport personnel and limited quantities of explosives in the cargo compartments of vehicles when required? (AFMAN 91-201) |  |  |  |  |
| 4.6. Are vehicles inspected prior to transporting explosives and do they meet the minimum guidelines? (AFMAN 91-201) |  |  |  |  |
| 4.7. Are explosive loaded vehicles and material handling equipment (MHE) chocked when parked and the driver is not behind the wheel? (AFMAN 91-201) |  |  |  |  |
| 4.8. Are explosives properly secured prior to transport? (AFMAN 91-201) |  |  |  |  |
| 4.9. Is only authorized maintenance being performed on explosive laden vehicles? (AFMAN 91-201) |  |  |  |  |
| 4.10. All vehicles to be used for off-base shipments of explosives will be inspected by shipping activities before and after loading for compliance with safety regulations. Are DD Form 626s completed according to DTR 4500.9R, Part 2, Chapter 204? (AFMAN 91-201) |  |  |  |  |
| 4.11. When transporting items containing electro- explosive device (EEDs) are the requirements being met? (AFMAN 91-201) |  |  |  |  |
| 4.12. Are seats and safety belts provided for all personnel? (AFI 91-207) |  |  |  |  |
| 4.13. Are personnel aware of the primary and alternate explosive routes on base? (AFMAN 91-201) |  |  |  |  |

| 4.14. Are explosives handled by only trained personnel? (AFI 91-201) |  |  |  |  |
| --- | --- | --- | --- | --- |
| 4.15. Is smoking prohibited within 50 feet of any conveyance or material handling equipment loaded with explosive items? (AFMAN 91-201) |  |  |  |  |
| 4.16. Do explosives loaded / unloaded from a motor vehicle while the engine is running meet the requirements listed? (AFMAN 91-201) |  |  |  |  |
| 4.17. Do containers prevent item to item contact and are they marked to identify their contents? (AFMAN 91-201) |  |  |  |  |
| 4.18. Are munitions / containers tumbled, dragged, dropped, thrown, rolled, or walked? Containers designed with skids may be pushed or pulled for positioning. (AFMAN 91-201) |  |  |  |  |
| 4.19. Are explosives laden vehicles not left unattended except as required in the event of an electrical storm (see Section 7H), unless parked in the munitions storage area or flightline munitions holding area? (AFMAN 91-201) |  |  |  |  |
| 4.20. Are supervisors knowledgeable of steps to take when an abnormal condition exists? (AFMAN 91-201) |  |  |  |  |

Attachment 6

CIVIL ENGINEER WORK ORDER REQUEST PROCEDURES

A8.1. For all safety inspection write-up(s), findings, or discrepancies which required CE support, please have your facility manager closely follow the below instructions and follow up actions.  Please assure your facility manager completes the below instructions BEFORE the aforementioned 30 calendar day suspense.

A8.2. Complete an AF FORM 332 (Base CE Work Request) to include Blocks 1-3 and 5-16 (Block 4 shall be filled in later).  Be sure to clearly specify and thoroughly describe the work required in Block 8 (Description of Work to be Accomplished).  Please include sketch or plan when applicable or to better communicate the desired work.

A8.3. When completing Block 9 (Brief Justification for Work to be Accomplished), please reference the specific safety inspection name, date, safety write-up discrepancy tracking number(s), the specific safety inspection write-up, finding, or discrepancy verbiage, AFI or reference numbers as required, and any related risk assessment codes (RAC).  Of note, one AF FORM 332 may account for multiple reported discrepancies as long as the discrepancies are all referenced in Block 9, all the work requested is identical in scope, and contained within the same facility.

A8.4. E-mail your completed AF FORM 332 (minus Block 4) to the 36th Civil Engineer Squadron Customer Service office at 36ces.service@us.af.mil.  Of note, you may also hand-walk the AF FORM 332 to the CE Customer Service office in Bldg 18001.  The AF FORM 332 shall then be reviewed by the CE Customer Service office for information completeness.  If the CE Customer Service office deems the AF FORM 332 has adequate information, a work order tracking number shall be assigned to it.  The CE Customer Service office shall then e-mail the work order tracking number back to you within approximately two duty days.  Of note, if the CE Customer Service office deems the AF FORM 332 does not have adequate information, the AF FORM 332 shall be returned to you for corrections.  Please contact the CE Customer Service office at DSN 366-2915 or DSN 366-2918 if you have any questions.

A8.5. Once you have received the work order tracking number from the CE Customer Service office, enter the number in the AF FORM 332's Block 4 (work request number) and e-mail with the updated AF FORM 332 to the 36th Wing Occupational Safety office at  36WG.SEG2@us.af.mil.  Ensure the following format is used in the e-mail subject line: "ACTION: WO #XXXXXXX for 36 (insert squadron) Critical Safety Write-Up #XXXXXX".  If you have any questions, please contact CE Customer Service office at DSN 366-2915/2918.

A8.6. As a reminder, a status update of all open safety inspections write-ups, findings, or discrepancies are required to be submitted to the 36th Wing Occupational Safety office at  36WG.SEG2@us.af.mil  30 days after inspection report is released to the unit.  This status report is required to be submitted to the 36th Wing Occupational Safety office every 30 days until all writeups, findings, or discrepancies are closed.

Attachment 7

OCCUPATIONAL SAFETY AND HEALTH ADMINISTRATION (OSHA) RECEPTION PROCEDURES

Figure A7.1.  36th Wing Inspections and Investigations Reception Policy Memo

Attachment 8

GUAM WATER SAFETY MEMORANDUMS

Figure A8.1.  36th Wing Guam Water Activity Restrictions

Figure A8.2.  Joint Region Marianas Off-Limits Locations and Restricted Water Activities on Guam